LM60S LED SERIES(Special Instructions)

- LCD display board contains the information of the driver board, after changing the display board and motor driver board, “Restore default setting” is needed.

- In the process of reset, long press the touch screen or “OK” button for 5 seconds to interrupt the reset.

- Press “OK” button or touch screen to interrupt reset process into Test Mode(The functions in Test Mode in tuning process will make the problems more visible.)

- DMX address is 512, return to the Main Interface, long press the “512” on the screen or “OK” for about 5 seconds to show/hide logo.

- Gobo wheel and color wheel have zero calibration function. Be caution when installing the Hall, channel 0 means using rest calibration to fine tuning , better with magnet matched, zero calibration will not work if the adjustment range of the gobo wheel and color wheel is out of +20. If magnet is matched, user turn channel into 0 when finding the gobo wheel or color wheel of some lamp is out of step to correct the wrong. Turn off the zero calibration functions in Advanced setting item named “Zero calibrations of pattern color”

- Signal lights:

  - The blink of ERR red light means error, check the detail in “Information”->”System error information”

  - The on status of DMX blue light mean received the DMX signal, off means no.

  - The light in motor driver board: 1. Blinking as 1-s intervals means receiving the uart signal sent from display board; 2. Blinking as 2-s intervals means no uart signal; 3. Always on or always off means something is wrong about the motor driver board.